JS-TP10-2

## 1. Product Title

Jumpstar JS-TP10-2 Multi-Mode Single-Frequency GNSS Positioning Module - Ultra-Compact Size, Low Power, Strong Anti-Interference Micro Positioning Solution

## 2. Short Description

- Single-Band Full Constellation Support: Simultaneous reception of GPS, BDS, GLONASS, Galileo, QZSS, SBAS for global stable positioning.

- Ultra-High Sensitivity: Tracking sensitivity up to -165dBm, reliable positioning in weak-signal and complex environments.

- 47 High-Speed Channels: Fast satellite signal acquisition and locking for efficient positioning.

- 10Hz High-Speed Update: Max navigation update rate up to 10Hz for real-time dynamic positioning.

- Ultra-Compact Design: 10.5×9.7×2.3mm mini size, perfect for space-constrained embedded devices.

## 3. Product Details

### A. Overview

The JS-TP10-2 is a low-power, cost-effective multi-mode single-frequency GNSS positioning module built on a high-performance SOC positioning engine. It supports concurrent reception of six global satellite systems, equipped with 12 multi-tone active interference cancellers and indoor/outdoor multipath detection & compensation technology to ensure stable positioning in challenging signal environments. Integrated with TCXO, SAW and RTC, it supports AGPS, standard NMEA 0183 protocol and 1PPS high-precision timing. With ultra-compact size, it is widely used in automotive, maritime, logistics, drones, robots and emergency rescue scenarios.

### B. Key Features & Benefits

- High-Performance GNSS SOC: Professional positioning engine for stable and efficient operation.

- 47 Channel Configuration: Fast satellite signal capture and tracking for quick positioning.

- Ultra-High Sensitivity: Tracking -165dBm, cold start -148dBm for extreme weak-signal adaptation.

- 10Hz High-Speed Update: Max navigation update rate up to 10Hz (default 1Hz) for real-time data output.

- Strong Anti-Interference: 12 multi-tone active interference cancellers + multipath detection & compensation.

- Full Constellation Support: Compatible with all mainstream GNSS systems for global coverage.

- AGPS Fast Positioning: Hot start in 1s, cold start in 24s for instant positioning response.

- All-In-One Integration: Built-in TCXO, SAW, RTC, simplifies peripheral circuit design.

- Rich Protocol Support: Compliant with NMEA 0183 V3.01/V4.10 (ASCII) for easy system docking.

### C. Technical Specifications

| Parameter | Specification |
| --- | --- |
| Receiver Type | Multi-mode single-frequency GNSS positioning module |
| Supported Constellations & Bands | GPS/QZSS L1C/A; BDS B1I/B1C; GLONASS L1; Galileo E1; SBAS(WAAS/EGNOS/MSAS/GAGAN) |
| Tracking Channels | 47 channels |
| Sensitivity | Tracking & Navigation: -165dBm; Reacquisition: -148dBm; Cold Start: -148dBm |
| Time to First Fix | Cold Start:24s; Hot Start:1s (all satellites ≥-130dBm) |
| Positioning Accuracy (CEP 50%) | Horizontal:2.0m; Vertical:3.75m (open sky, 24h static) |
| Time Accuracy | 1PPS:10ns; Time pulse frequency:1Hz |
| Speed Accuracy | GNSS:0.1m/s; SBAS:0.05m/s |
| Operational Limits | Acceleration≤4g; Altitude≤10000m; Speed≤100m/s |
| Baud Rate | 4800-921600bps (Default:115200bps) |
| Max Update Rate | 10Hz (Default:1Hz) |
| Supported Protocols | NMEA 0183 V3.01/V4.10 (Input/Output, ASCII) |
| Antenna Type | Supports passive/active antenna; Active gain:15~30dB |
| Pin Interfaces | TXD/RXD(UART), PPS, VCC/VBACK(Power), RESET, RF_IN, VCC_RF, SDA/SCL(I2C) |
| Power Supply Voltage | VCC:2.83.6V (Typical:3.3V); Backup:1.623.63V |
| Operating Current | Acquisition:3138mA@3.3V; Tracking:3035mA@3.3V; Backup:11uA@3.3V |
| Digital I/O Voltage | 2.8~3.3V |
| Operating Temperature | -40℃ ~ +85℃ |
| Storage Temperature | -40℃ ~ +105℃ |
| Humidity | 95% |
| Mechanical Dimensions | 10.5mm × 9.7mm × 2.3mm ±0.3mm |
| Compliance | RoHS compliant |

## 4. Typical Applications

· Automotive Applications & Car Navigation

· Maritime Navigation & Fleet Management

· Intelligent Logistics Dispatching & Asset Tracking

· Formation UAVs & Drones

· Intelligent Robots & Autonomous Equipment

· Ground Vehicle Tracking & Urban Intelligent Traffic

· Emergency Rescue & Disaster Relief Positioning

· Personal Travel & Wilderness Adventures

## 5. Technical FAQ

Q1: What is the package size of JS-TP10-2?

Ans: Ultra-compact size:10.5×9.7×2.3mm±0.3mm, ideal for miniaturized and space-limited devices.

Q2: What is the sensitivity and positioning speed?

Ans: Tracking sensitivity up to -165dBm; hot start 1s, cold start 24s for fast positioning.

Q3: What anti-interference functions does it have?

Ans: Built-in 12 multi-tone active interference cancellers + indoor/outdoor multipath detection & compensation.

Q4: What protocols and interfaces are supported?

Ans: Protocol: NMEA 0183 V3.01/V4.10; Interfaces: UART, I2C, 1PPS, RF antenna, power supply.

Q5: What is the power supply and operating temperature range?

Ans: Power supply 2.83.6V (typical 3.3V); operating temperature -40℃+85℃, industrial-grade reliability.